# DATA ANALYTICS OF GAS SENSOR FOR HEALTH CARE APPLICATIONS

# A BTP Report

# by

- K. Gnanaprakash (S20220020288)

- E. Akash (S20220020270)

BTP Code:- B25PD02

INDIAN INSTITUTE OF INFORMATION TECHNOLOGY SRICITY

2nd Semester Report

INDIAN INSTITUTE OF INFORMATION TECHNOLOGY SRICITY

CANDIDATE’S DECLARATION

I hereby certify that the work which is being presented in the BTP entitled

“Data Analytics of health care applications using gas sensors” in the partial fulfillment of the requirements for the award of the degree of B. Tech and submitted in the Indian Institute of Information Technology SriCity, is an authentic record of my own work carried out during the time period from January 2025 to December 2025 under the supervision of Dr.Priyanka Dwivedi, Indian Institute of Information Technology SriCity, India. The matter presented in this report has not been submitted by me for the award of any other degree of this or any other institute.

Signature of the student with date

Kovuru Gnanaprakash 22-12-2025

−−−−−−−−−−−−−−−−−−−−−−−−−−−−−−−−−−−−−

This is to certify that the above statement made by the candidate is correct to the best

of my knowledge.

Signature of BTP Supervisor with date

(Prof. XYZ)

Contents

INTRODUCTION 	4

LITERATURE SURVEY 	5

METHODOLOGY 	7

RESULTS 	8

CONCLUSION 	9

## 1. Abstract

This study presents an experimental and analytical comparison of gas sensing performance for healthcare applications using Silicon Nanowire Sensors (SiNWS) and SnO₂ sensors. Both sensors were exposed to controlled concentrations of acetone vapor, and variations in resistance were measured to analyze sensing characteristics. The captured data was further evaluated using machine learning models to determine prediction accuracy and sensor reliability. Results demonstrated that the SnO₂ sensor provided greater stability and accuracy, while SiNWS showed significant response variation with nonlinear behavior. The combined experimental and data-driven approach highlights the potential of gas sensors for VOC monitoring in hospital environments, supporting real-time air quality detection and future diagnostic applications.

## Introduction

Volatile Organic Compounds (VOCs) are frequently detected in clinical and industrial healthcare environments due to disinfectants, solvents, and cleaning agents. Prolonged exposure to these gases can harm patients and medical personnel. Sensors capable of detecting VOCs at low concentrations are therefore essential in hospitals for safe air quality management. This project investigates the resistance-based response of SiNWS and SnO₂ sensors when exposed to acetone, evaluating how accurately these sensors detect changes over time. Machine learning models were applied to captured resistance data to improve VOC prediction and understand sensor behavior. The study aims to identify a reliable sensing platform suitable for future real-time healthcare implementation.

Related Work

| Title of the paper | Objective | Key findings |
| --- | --- | --- |
| Sensitive H2 gas sensorsbased on SnO2 nanowires | Investigate how different functional groups on SiNWs affect their sensitivity and selectivity towards VOCs. | Understanding surface functionalization aids in optimizing SiNW gas sensors for detecting diseasespecific VOCs in healthcare applications |
| Functionalized SiNW Arrays for Human VOCs | Functionalized SiNW Arrays for Human VOCs | Enhances healthcare VOC analysis; data analytics can classify disease markers. |
| Review nanowire and nanotube-based sensor technologies for air pollutants and VOCs. | Review nanowire and nanotube-based sensor technologies for air pollutants and VOCs. | Supports understanding of nanowire sensor mechanics in healthcare settings. |

Problem statement and contribution

## Problem Statement

VOC exposure continues to present serious health and safety concerns in hospitals. Detecting low-level VOC concentration accurately and consistently remains a challenge due to sensor instability, noise, drift, and cross-sensitivity. Traditional electrical sensing techniques alone may not provide robust prediction capability for real-world usage.

Contribution:

K.Gnanaprakash:  Collected sensor data sno2 (100ppm,200ppm) and sinws for

(100ppm,200ppm) using keithely software and plotted graphs in origin and applied machine learning algorithms like Random forest, linear regression.

E.Akash: Collected sensor data sno2 (400ppm) and sinws for(400ppm,500ppm) using keithely software and plotted graphs using origin.

# Methodology:

The experiment exposes acetone vapour to both SiNWS and SnO₂ gas sensors under identical laboratory conditions. Each sensor is placed in a probing station, where cotton serves as the VOC transfer medium, and acetone volumes are calculated using a syringe to achieve 100 ppm, 200 ppm, and 400 ppm exposure levels. After each injection, the setup is allowed to stabilize for 15 minutes to ensure complete VOC–sensor interaction before the next concentration is introduced. Throughout the process, resistance changes are recorded in real time using a Keithley 2614 source meter through KickStart software. To analyse the response patterns, machine learning models were implemented, including Linear Regression for capturing direct relationships between sensor parameters and Support Vector Regression (SVR) for analysing nonlinear behaviour, enabling deeper insight into sensor performance and prediction accuracy.

# Experimental results

0

2000

4000

1500

2000

2500

3000

3500

4000

4500

5000

Resistance

Time

sno2-Acetone

ppm

100

200

ppm

400

ppm

500

1000

3400

3600

3800

4000

4200

4400

Resistance(



)

Time(s)

50

ppm

100

m

pp

200

ppm

400

ppm

Conclusion

The study confirms that both SiNWS and SnO₂ sensors are responsive to acetone vapor exposure. Comparative evaluation using machine learning shows that SnO₂ provides higher accuracy and stability, making it more suitable for healthcare VOC monitoring. SiNWS demonstrated moderate performance, but results indicate strong potential for improvement through nonlinear analytical modeling.

Furthermore, the outcomes highlight the importance of integrating experimental sensing with data–driven analytics to enhance VOC detection reliability. The strong predictive capability displayed by the SnO₂ model suggests that nanoscale metal– oxide sensing platforms, when supported by machine learning, can be optimized for real-time hospital   air  - quality monitoring .